BASIC ELECTRICAL ENGINEERING

Chapter 1

General Electric System

- Find the velocity of charge leading to 1 A current which flows in a copper conductor of cross-section 1 cm2 and length 10 km. Free electron density of copper = 8.5 × 1028 per m3. What will be the velocity of charge and how long will it take the electric charge to travel from one end of the conductor to the other? [0.735 μm/s, 431 years]

- The resistivity of a metal alloy is 50 × 10−8 ohm-m. A sheet of material 15cm long, 6 cm wide and 0.014 cm thick. Calculate the resistance in the direction:

  - Along the length and [8.93 mili ohm]

  - Along the thickness. [7.77 × 10-9 ohm]

- An aluminum wire 7.5 m long is connected in a parallel with a copper wire 6 m long. When a current of 5 A is passed through the combination, it is found that the current in the aluminium wire is 3 A. The diameter of the aluminium wire is 1 mm. Determine the diameter of the copper wire. Resistivity of copper is 0.017 μΩ-m ; that of the aluminium is 0.028 μΩ-m. [0.569 mm]

- One km of wire having a diameter of 11.7 mm and of resistance 0.031 ohm is drawn so that its diameter becomes 5 mm. What will be its new resistance? [0.928 ohm]

- A lead wire and an iron wire are connected in parallel. Their respective specific resistances are in the ratio 49 : 24. The former carries 80 percent more current than the latter and the latter is 47 percent longer than the former. Determine the ratio of their cross sectional areas. [0.4]

- The resistance of the wire used for telephone is 35 Ω per kilometre when the weight of the wire is 5 kg per kilometre. If the specific resistance of the material is 1.95 × 10−8 Ω-m, what is the cross-sectional area of the wire ? What will be the resistance of a loop to a subscriber 8 km from the exchange if wire of the same material but weighing 20 kg per kilometre is used ? [55.7 × 10–8 m2]

Effect of Temperature on resistance

- A copper conductor has its specific resistance of 1.6 × 10−6 ohm-cm at 0°C and a resistance temperature coefficient of 1/254.5 per °C at 20°C. Find (i) the specific resistance and (ii) the resistance - temperature coefficient at 60°C.

- A potential difference of 250 V is applied to a field winding at 15°C and the current is 5 A. What will be the mean temperature of the winding when current has fallen to 3.91 A, applied voltage being constant. Assume α15 = 1/254.5.  [86°C]

- Two coils connected in series have resistances of 600 Ω and 300 Ω with temperature coefficient of 0.1% and 0.4% respectively at 20°C. Find the resistance of the combination at a temperature of 50°C. What is the effective temperature coefficient of combination?

- Two wires A and B when connected in series at 0°C, the resistance of B is 3.5 times that of A. The resistance temperature coefficient of A is 0.4% and that of the combination is 0.1%. Find the resistance temperature coefficient of B. [0.0143 %]

- Two resistors made of different materials have temperature coefficients of resistance α1=0.004 per celcius and α2=0.005 per celcius at 0 °C are connected in parallel and consume equal power at 15 °C. What is the ratio of power consumed in resistance R2 to that in R1 at 70 °C? [0.962]

- A coil is connected across a constant dc source of 120V. It drawa a current of 12A at room temperature of 25°C. After 5 hours of operation, its temperature rises to 65°C and current reduces to 8A. Calculate:

  - Current when its temperature has increased to 80°C    [Ans: 7.111A]

  - Temperature coefficient of resistance at 30°C      [Ans: 0.01176/°C]

- A 230V metal filament lamp has its filament 50cm long with cross sectional area of 3 × 10−6 cm2. Specific resistance of the filament metal at 20°C  is 4 × 10−6 ohm cm. if the working temperature of the filament is 2000°C, find the wattage of the lamp. Temperature coefficient of resistance of the filament material at 20°C is 0.0055 /°C. [66.74W]

- The filament of a 240 V metal-filament lamp is to be constructed from a wire having a diameter of 0.02 mm and a resistivity at 20°C of 4.3 μΩ-cm. If α = 0.005/°C, what length of filament is necessary if the lamp is to dissipate 60 watts at a filament tempt. of 2420°C ?

- The temperature rise of a machine field winding was determined by the measurement of the winding resistance. At 20°C, the field resistance was 150 ohm. After running the machine for 6 hrs at full load, the resistance was 175 ohm. The temperature coefficient of resistance of the copper winding is 4.3 × 10−3 /K at 0°C. Determine the temperature rise of the machine. [42.098°C]

- The coil of a relay takes a current of 0.12 A when it is at the room temperature of 15°C and connected across a 60-V supply. If the minimum operating current of the relay is 0.1 A, calculate the temperature above which the relay will fail to operate when connected to the same supply. Resistance-temperature coefficient of the coil material is 0.0043 per°C at 6°C. [63.31 deg. Cel.]

- A semi-circular ring of copper has an inner radius 6 cm, radial thickness 3 cm and an axial thickness 4 cm. Find the resistance of the ring at 50°C between its two end-faces. Assume specific resistance of Cu at 20°C = 1.724 × 10−6 ohm-cm and resistance tempt. coeff. of Cu at 0°C = 0.0043/°C.