Tutorial: 6
Subject: Basic Electrical Engineering

- An alternating current of sinusoidal waveform has an rms value of 10A. What is the maximum value of the current?		[Ans: 14.142A]

- The following three sinusoidal currents flow into a junction:

i1=32 Sin(ωt), i2=5 2 Sin (ωt+300) and i3=62 Sin(ωt-1200)

Find the time expression for the resultant sinusoidal current which leaves the junction. Draw the phasor diagram.	[Ans: 7.214 Sin(ωt-31.910)

- If V = 452 - j45, Z1 = 2.52 - j2.52, Z2 = 7.5 + j7.53. Find V.Z1Z2. 	[Ans: 30∠-450]

- If v = 14.14 Sin(ωt+30) V and i=11.31 Sin(ωt-30)A. Find

  - Maximum Value of each. 	[Ans: Vm=14.14, Im=11.31]

  - The rms value of each. 		[Ans: Vrms=10, Irms=8]

  - The phasor expression in polar and rectangular form.	Ans: V = 10∠30 = 8.66+j5I=8∠-30=6.928-j4

- Find the time taken for a sinusoidal alternating current of maximum value 20A to reach 15A for the first time if the current starts from zero. The supply frequency is 40 Hz.	[Ans: 3.37 mS ]

[Hint: i=20 Sinωt. When i=15 A, 15=20 sinωt => ωt=sin-1(15/20)= 48.590 =0.848 rad. Now,   ω=2πf=251.32 rad/sec. Thus t= 0.848/251.32 sec = 3.37 milliseconds]

- A sinusoidal alternating voltage of peak value 24 V is applied to a circuit resulting a current of maximum 16 A. The current is also sinusoidal and lags the voltage by 450. Draw the voltage and current waveforms in proper phase relationship over 1 complete cycle.

- Calculate the average, rms value, form factor and peak factor for the following saw tooth waveform.	[Ans: Vavg = 25V, Vrms = 28.8675V, Form factor = 1.15, Peak factor = 1.73]

- Determine the average and rms values of the voltage for sinusoidal voltage waveform as shown below.	[Ans: Vavg =27.169 V and Vrms = 47.67 V]

0

v

Vm=100V

π/4

π

2π

9π/4

3π

θ

- Determine the average and rms values of current for current waveform as shown below.		[Ans: Iavg = - 2.5 A and Irms = 8.165 A]

- Calculate the average, rms values, form factor and peak factor of the following waveform.	[Ans: Vrms = 15.2, Vavg = 15, Form Factor = 1.0133, Peak Factor = 1.3158]

- The equation relating the current in a circuit with time is i=141.4Sin377t.

  - Find the values of Irms.

  - Frequency

  - Instantaneous value of the current I in 3 millisecond.	[Ans: 99.985V, 60Hz, 127.944V]

- A sinusoidal voltage is applied to three parallel branches yielding branch currents, i1=14.14 Sin(ωt-450), i2=28.3 Cos(ωt-600) and i3=7.07 Sin(ωt+600). Find the complete time expression fo the source current. Draw the phasor diagram in terms of effective values. [Ans: 39.4 Sin(ωt+15.1)]