Subject: Basic Electrical Engineering

Tutorial: 7

Single Phase Circuit Analysis:

- 230V, 50Hz ac is applied to a coil of 0.06H inductance, 2.5Ω resistance and 6.8μF capacitor in series. Calculate the current and phase angle of the circuit. Also calculate the voltage across each of the circuit components.	 [Ans: 0.5119∠89.680, VR=1.279V, VL=9.649V, VC = 239.2V]

- The following loads are connected to a common supply.

Load A 150 KVA at 0.707 pf lagging

Load B 100 KW at unity pf.

Load C 75 KW at 0.8 pf lagging.

Load D 50 KW at 0.6 pf lagging.

Determine the total active, reactive and apparent power and overall power factor.	[Ans: 331.05KW, 269.82 KVAR, 427.08 KVA, 0.775]

- A single phase ac voltage of 240V is applied to a series circuit whose impedance is 10∠600 Ω. Find R, X, P, Q and the power factor of the circuit. If a capacitor is connected in parallel with the above circuit, and if the capacitor supplies 1250 VAR, find the P and Q supplied by the 240V source and find the resultant power factor. Draw the power triangle.	[Ans: R = 5 Ω, X = 8.66 Ω, P = 2880 W, Q = 4988.3 VAR; Once the capacitor is connected, P = 2880 W and Q = 3738.30 VAR ]

- A voltage source Ean= 120∠300 and the current through it is Ian=10∠600 A. Find the values of P and Q and state whether the source is delivering or receiving. [Ans: 1039.23 W, 600 VAR, Current leads the voltage, so the load is supplying reactive power and the source is receiving reactive power.]

- For the circuit shown

  - Find the effective value of the current delivered by the source. Write down the expression for the instantaneous value of the current.

  - Draw a carefully labelled phasor diagram showing the source voltage, source current, voltage Vab and Vbc.

  - Compute the power supplied by the source and the line pf.

[Ans: 10.289∠84.10, i(t) = 14.55 Sin(ωt+84.10), P = 126.915W, pf = 0.1028 (lead)]

- Calculate magnitude and phase of the current from the source in the following ac parallel circuit. [Ans: 2.918∠-53.3480 A]

- Two coils A and B are connected in series across a 240V, 50Hz supply. The resistance of A is 5Ω and the inductance of B is 0.015H. If the input from the supply 3KV and 2KVAR. Find the inductance of A and the resistance of B. Calculate the voltage across each coil.

- A circuit comprises of three branches A, B and C. A and B are in parallel while C is in series with this parallel combination of A and B. The impedance of the branches are ZA = (4+j3)Ω, ZB = (10-j7)Ω and ZC = (6+j5)Ω . If the voltage 230V @ 50Hz is applied to the circuit, calculate

  - Total power factor of the whole circuit.

  - Draw the complete phasor diagram.

- A circuit is composed of a resistance of 9 Ω and series connected inductive reactance of 12 Ω. When a voltage is applied to the circuit, the resulting steady current is found to be i = 28.3 Sin377t. What is the value of the complex impedance? Determine the time expression for the applied voltage. Find the value of inductance in Henry.	[Ans: 15∠53.130; 424.2 Sin(377t+53.130); 0.0318 H]

- An rms voltage of 100∠ 00 is applied to the series combination of Z1 and Z2 where Z1=20∠ 300. The effective voltage drops across Z2 is known to be 40∠ -300V. Find the reactive component of Z2. 	[Ans: 33.3Ω]

- A 50Hz, 220 V is applied to a 0.637 inductor. (i) Write the equation for the applied voltage and current through the inductor. (ii) Plot the voltage and current waveforms. (iii) Draw the phasor diagram for voltage and current. 	[Ans: v(t)=311 Sin314t, i(t)=1.55 Sin(314t-π/2)].

- A 50Hz, 220V, voltage is applied to a capacitor of value 0.636μF. (i) Write equations for instantaneous voltages and current as functions of time t. Assume voltage at time t=0 to be zero and going positive. (ii) Plot v and I against angle ωt. (iii) Draw the phasor diagram for voltage and current. 	[Ans: v(t)=311 Sin(314t), i(t) = 62.2X10-3 Sin(314t+π/2)]

- A sinusoidal voltage v = 230∠150 of frequency 50 Hz is applied to a series R-L circuit consisting of R=4Ω and L=0.05H. Calculate (i) the rms current and its phase angle. (ii) Power factor (iii) average power drawn by the circuit (iv) Reactive power and apparent power (v) Q-factor of the coil. 	[Ans: 14∠ -60.710 A; cos 75.71 (lagging), 805 watts; 3163 VAR, 3264 VA]

- A 200V, 50Hz, supply is applied to a coil of resistance R=10 Ω and inductance 0.2 H, Calculate (i) the reactance and impedance of the coil (ii) current and its phase angle relative to applied voltage. Draw the phasor diagram. 	[Ans: 62.82Ω; 63.61Ω; 3.144∠-810, VR=31.44V, VL=197V]

- A voltage v = 141.4 Sin(314t) is applied to a coil having inductance of 0.1H and resistance 10Ω. Find (i) expression for the instantaneous current. (ii) rms value of the voltage and current (iii) power factor (iv) power consumed in the coil. 	[Ans: i(t) = 4.29 Sin(314t-72.330), 100V, 3.03A, cos72.330(lagging), 91.8 Watts].

- A 50Hz, sinusoidal voltage (40+j30) volt is applied to a series R-L circuit resulting in sinusoidal current (4+j1) Amperes. Calculate (i) the impedance of the circuit. (ii) power consumed in the circuit (iii) power factor of the circuit. 	[Ans: Z = (11.17+j4.72)Ω, 190 W, Cos22.90(lagging)]

- An inductive coil having resistance 10Ω takes a current of 2.2A when connected to 110V, 60Hz supply. If the coil is connected to 50Hz supply, Calculate (i) current (ii) power (iii) power factor. Draw the phase diagram when 50Hz supply is connected. 	[Ans: 2.61A, 68.1 W, 0.2378(lagging), VR=26.1V, VL=106V]

- When connected to 200V, 50Hz supply, a certain coil draws current of 13A. If the frequency of the supply is changed to 60Hz, the current decreases to 11A. Calculate the resistance and inductance of the coil. [Ans: R=2.14Ω, L=0.04655H]

- A coil having resistance of 10 Ω and inductance of 0.2H is connected in series with coil B having resistance of 30Ω and inductance of 0.1H. The two coils in series are fed from 200V, 50Hz supply. Determine (i) the voltage across each coil. (ii) Power dissipated in each coil. (iii) Power factor of the circuit as a whole.	[Ans: VA=136.66V, VB=93.3V, PA=46.14W, PB=138.4W, Cos670]

- A coil draws 11A and 800 W from 220V, 50Hz supply. Determine (i) the impedance of the coil. (ii) the power factor and (iii) reactive and apparent power.	[Ans: Z= (6.61+j18.8)Ω, Cos70.60(lagging), 2282VAR, 2420VA]

- A coil A draws current of 10A and power of 200W when driven by 110V, 60Hz supply. Another coil B draws current of 11A and power 400W when driven by the same source. What current and power will be drawn from the same source by coils A and B connected in series. [Ans: 5.27A, 147.2W]

Power Factor Improvement in single phase AC circuits:

- An inductive load of 4KW at a lagging power factor of 0.8 is connected across a 220V, 50Hz supply. Calculate the value of the capacitance to be connected in parallel with the load to bring the resultant power factor to 0.95 lagging.	[Ans: 110.86μF]

- A 100 KW load at 0.8 lagging power factor is being supplied by a 220 V, 50Hz source. Calculate the reactive power drawn from the source. If a capacitor connected parallel to the load improves its power factor to 0.9, find the capacitance of the capacitor. Also calculate the current drawn from the source before and after connecting the capacitor.	[Ans: Q=75kVAR, Ibefore=568.18A, QC=26.572kVAR, C=1747.547 μF, Iafter=505.05A]

Three Phase Circuit Analysis:

- A 220V, 3-Φ voltage is applied to a balanced delta connected 3-Φ load of phase impedance (15+j20) Ω. Calculate:

  - The phase voltages	[Ans: vR=220∠00, vY=220∠-1200, vB=220∠1200]

  - The phasor current in each line	[Ans: i1=15.24∠-83.130, i2=15.24∠156.870, i3=15.24∠36.870]

  - The power consumed per phase		[Ans: 1161.603 W]

  - Draw the phasor diagram

  - What is the phasor sum of three line currents? 	[Ans: 0]

- For the delta connected load, find the phase currents, line currents, power (active, reactive and apparent) in each phases. Also determine the total active power consumed.

[Ans: Phase Currents: iR=8.8∠-53.130, iY=7.54∠-89.030, iB=8.8∠83.130

Line Currents: i1=16.33∠-750, i2=5.176∠-174.470, i3=16.301∠86.750

Powers:	R phase: PR=2323.205 W, QR=3097.595 VAR, SR=3872 VA

Y phase: PY=2844.63 W, QY=-1707.2 VAR, SY=3317.6 VA

B phase: PB=3097.595 W, QB=2323.205 VAR, SB=3872 VA

Total Active power P=8265.43 W]

- Three impedances of (10+j10)Ω, (12+j12)Ω and (2+j2)Ω are connected in delta to a 3-Φ system with line voltage 400V. Calculate all the phase currents, line currents, active powers, reactive powers and apparent power.

[Ans: 	Phase Currents: iR=28.28∠-450, iY=23.57∠-1650, iB=141.42∠750

Line Currents: i1=157.47∠-96.050, i2=44.96∠1620, i3=154.56∠67.410

Powers:	 R phase: PR=7998.79 W, QR=7998.79 VAR, SR=11312 VA

Y phase: PY=6666.602 W, QY=6666.602 VAR, SY=9428 VA

B phase: PB=39999.61 W, QB=39999.62 VAR, SB=56568 VA]

- In a 3-phase, 4 wire Wye connected system the phase voltage VPh=200V and its frequency is 60 Hz. The load impedance components are R1=100 Ω, R2=100 Ω, C2=66.3 μF, R3=100 Ω, L3=159.2 mH. Calculate the three line currents and the neutral current.	[Ans: iR=2∠00, iY=1.85∠-98.20, iB=1.71∠89.040, iN=1.77∠-3.930]

- A 415V, 3 phase, 50 Hz induction motor takes 50 kW power from supply mains at 0.72 power factor lagging. Determine the KVAR rating of capacitor bank to improve the power factor to 0.9 lagging. Calculate the capacitance per phase required if the capacitor bank is connected in:

(i) delta	[Ans: 147.86 μF]

(ii) star [Ans: 443.57 μF]

[Ans: QC= 24kVAR]

- The power input to a motor is measured by two wattmeters, which indicate 40kW and 50kW respectively. If the power factor of the motor is changed to 0.8 leading, determine the reading of two wattmeters. The total input power remains the same.	[Ans: W1=64.35kW, W2=25.65kW]

- The input power to a three-phase motor was measured by two wattmeter method. The readings were 10.4kW and -3.4kW and the voltage was 400V. Calculate (i) the power factor (ii) the line current.	[Ans: 0.45 (lag), IL=22.4 A]

- Phase voltage and current of a star-connected inductive load is 150V and 25A. Power factor of load is 0.707 (lag). The power is measured using two wattmeters. Find the readings of wattmeters.	[Ans: W1=6273W, W2=1681W]